Lava Wrapper Manual (1.0)

Khanh Dang, Jie Chen, Saryu Fensin

Los Alamos National Laboratory

04/28/2022

- Introduction

  - Overview

The Lava Wrapper is a general-purpose calculator that provides a python interface to enable one-click calculation of the many common properties with lammps and vasp. The name Lava is derived from the “La” in Lammps and “va” in vasp. It provides a set of classes and functions to generate configurations, run lammps/vasp calculation, retrieve the output, postprocess and plot the results. All the above tasks are hard-coded into the script, without the need to call additional libraries.

  - Features

Lava Wrapper comprises of eight major python modules. The following flow chart gives an overall idea of the workings of Lava Wrapper. Lava_Wrapper.py provides the outmost layer of abstraction, where the users can specify the set of calculations to perform, invoking the corresponding functions in Lava_Calculator.py, which reads in configurations generated by Lava_Generator.py, submits lammps/vasp run, extracts and postprocess the output, invoking utility functions in Lava_Utility.py. In this process, two other modules, Lava_Lammps _Header.py and Lava_Vasp_Header.py, are responsible for generating Lammps input script and vasp INCAR file, respectively. Finally, the results are plotted with Lava_Plotter.py. The last module, Lava_Config.py is used for storing global settings.

Fig. 1  Flow-chart of Lava Wrapper.

The following is a summary of each module’s functionality:

Lava_Wrapper.py: The outmost wrapper where the users specify the type of calculations, and the parameters pertaining to these calculations.

Lava_Calculator.py: The core of Lava Wrapper, responsible for performing the various types of calculations through the following steps: generate the relevant input file, perform calculations, retrieve and postprocess the output, and plot the results.

Lava_Generator.py: Functions for the generation of various crystal structures and defect configurations. The following 8 bulk crystal structures are currently supported: Simple Cubic (SC), Body Centered Cubic (BCC), Face Centered Cubic (FCC), Hexagonal Close Packed (HCP), Diamond Cubic (DC), double HCP, A5_Beta_Sn, A15_Beta_W.  The following defect configurations: vacancy, interstitial, surface, stacking faults, twin faults can also be added to the generated crystal structures.

Lava_Utility.py: Utility functions for file management, data processing, etc.

Lava_Lammps_Header.py: Functions for generating lammps input script, depending on the type of calculation.

Lava_Vasp_Header.py: Functions for generating vasp input script, namely, the INCAR file, depending on the type of calculation.

Lava_Plotter.py: Functions for plotting the results.

Lava_Config.py: Specification the lammps executable, the lammps potential file, as well as the element, provided by the user. The first two files are only required for running the wrapper in lammps mode.

  - Types of Calculations

Currently, Lava Wrapper incorporates the following types of calculations. For some calculations the output a written to “Summary.dat” file that serves as the general output for Lava_Wrapper, and for other types of calculation where the results are tabulable, then it is written to a separate data file and plotted as well.

| Type of calculation | Summary .dat? | Separate data file + Plot? |
| --- | --- | --- |
| Lattice constant and cohesive energy | ✓ | ✓ |
| Cold curve (E_vs._Volume/Density) |  | ✓ |
| Elastic properties (elastic constant, bulk modulus, etc.) | ✓ |  |
| Volume-conserving/non-conserving Bain path (1D and 2D) |  | ✓ |
| Liquid radial distribution function with varying T |  | ✓ |
| Vacancy, interstitial formation energy | ✓ |  |
| Surface energy | ✓ |  |
| Thermal expansion (lattice constant vs temperature) |  | ✓ |
| (Generalized) stacking fault energy (1D and 2D) |  | ✓ |
| Melting point (2-phase method) | ✓ |  |

- Examples

  - Running the wrapper

To run Lava Wrapper, load python3 first and then issue the following command on the terminal:

python3 Lava_Wrapper.py mode

wherein the highlighted mode can either be Lammps or Vasp, depending on which mode you wish to run.

Example scripts and output of Lava Wrapper are given below, when invoked for Al using Mishin-Farkas potential [1]. This can be set up by specifying the lammps executable as well as potential file name in Lava_Config.py in the following way:

lammps_executable = 'lmp_mpi’

element = "Al"

potential_file = 'Al99.eam.alloy'

Make sure that the lammps executable and the potential file is in the same folder as the Lava Wrapper scripts.

  - Lattice constants and cohesive energy

def get_cohesive_energy(phase_list, NSW=100, IBRION=1, ISIF=3, lat_rep=[2,2,2], orz=[0,0,1], mode=None):

The following snippet in Lava_Wrapper.py calculates the lattice constants and cohesive energy of a given list of crystal structures specified in ‘phase_list’, the 11 crystal structures shown below are currently supported. In addition, the user can provide additional lammps data file named “user_provided.data” as an additional phase.

### Lattice constants and cohesive energy phase_list = ["SC", "BCC", "FCC", "HCP", "DC", "DHCP", "A5_Beta_Sn", "A5_Gamma_Sn", "A15_Beta_W","L12","9R"] Lava_Calculator.get_cohesive_energy(phase_list)

The output is written out to Lammps_latt.dat, as shown below. Where phase, alat, blat, clat, atoms_per_cell, Ecoh represent the crystal structure, lattice constant a, b/a ratio, c/a ratio, number of atoms per unit cell, and the cohesive energy. This file is read in to perform other types of calculations which require information regarding lattice constants and cohesive energy.

phase a b/a c/a atoms_per_cell Ecoh SC 2.7407 1.0000 1.0000 1 -2.9614 BCC 3.2354 1.0000 1.0000 2 -3.2545 FCC 4.0500 1.0000 1.0000 4 -3.3600 HCP 2.8191 1.7321 1.7539 4 -3.3322 DC 6.0465 1.0000 1.0000 8 -2.4691 DHCP 2.8458 1.7321 3.3503 8 -3.3439 A5_Beta_Sn 5.3506 1.0000 0.5191 4 -3.0359 A15_Beta_W 5.1467 1.0000 1.0000 8 -3.3469

All parameters and their specification:

| Parameters | Specification |
| --- | --- |
| phase_list | The list of phases to evaluate cold curve on. |
| NSW (vasp) | Maximum number of ionic steps, default = 100 |
| IBRION (vasp) | Tag for selcting minimization algorithm, default = 1 |
| ISIF (vasp) | Tag for principal DOFs allowed to change in relaxation, default = 3 |
| lat_rep | Unit cell lattice replication along X, Y and Z direction, default = [2,2,2] |
| orz | Z orientation, default = [0,0,1] |
| mode | The mode (Lammps or Vasp) specified by the user when invoking Lava Wrapper |

  - Cold curve

def get_cold_curve_V(phase_list, V_start=None, V_stop=None, npoints=None, NSW=1, IBRION=1, ISIF=2, lat_rep=[1,1,1], orz=[0,0,1], V_list=None, mass=None, mode=None, dirname=None, output_name=None, plot_axis_x="volume", plot_axis_y="energy"): def get_cold_curve_R(phase_list, alat_expanse=None, npoints=None, NSW=1, IBRION=1, ISIF=2, lat_rep=[1,1,1], orz=[0,0,1], mass=None, mode=None, plot_axis_x="volume", plot_axis_y="energy"):

The following snippet in Lava_Wrapper.py calculates the cold curve of a given list of crystal structures specified with phase_list. Cold curve is calculated based on volume per atom V (E_vs._V). The grid points range from V_start to V_stop, with npoints evenly spaced points.

### Cold curve based on V phase_list = ["SC", "BCC", "FCC", "HCP", "DC"] Lava_Calculator.get_cold_curve_V(phase_list, V_start=10, V_stop=30, npoints=30, mass=4.48, mode= mode)

All parameters and their specification:

| Parameters | Specification |
| --- | --- |
| phase_list | The list of phases to evaluate cold curve on |
| V_start | Start in the range of volume to be evalauted (inclusive), default = None |
| V_stop | End in the range of volume to be evalauted (inclusive), default = None |
| npoints | Number of points in the range specified by V_start and V_stop |
| V_list | An explicit list of volume to be evaluated |
| mass | Atom mass (in unit of 10-23 g) for the purpose of calculating density |
| dirname | Path to run the calculations, individually set if not specified |
| output_name | Name of the output data and jpeg file, individually set if not specified |
| plot_axis_x | X axis for the plot, "density" or "volume" or "energy" |
| plot_axis_y | Y axis for the plot, "density" or "volume" or "energy" |

Cold curve can also be evaluated based on lattice constant R (E_vs._R). In this case, the grid points range from alat - alat_expanse to alat + alat_expanse, where alat is the lattice constants of the corresponding phase, with npoints evenly spaced points. As compared to E_vs._V, the advantage of this scheme that the potential valley will always be captured.

The following snippet in Lava_Wrapper.py calculates the cold curve based on R:

### Cold curve based on R ## Expanse of lattice (from alat-alat_expanse to alat+alat_expanse) phase_list = ["SC", "BCC", "FCC", "HCP", "DC"] Lava_Calculator.get_cold_curve_R(phase_list, alat_expanse=0.5, npoints=30, mass=4.48, mode=mode)

The tabulated E vs. V and E vs. R output is written to EOS_V_Lammps.dat and EOS_R_Lammps.dat, respectively. The corresponding plots: EOS_V_Lammps.jpeg and EOS_R_Lammps.jpeg are shown below in Fig. 2.

(a)  (b)

Fig. 2  Cold curve generated by Lava Wrapper with: (a) E vs. V scheme, (b) E vs. R scheme.

  - Elastic constants

def get_elastic_properties(phase_list, NSW=100, IBRION=6, ISIF=3, NFREE=2, lat_rep=[2,2,2], orz=[0,0,1], mode=None, use_built_in_method=False):

The following snippet in Lava_Wrapper.py calculates the elastic constants of a given list of crystal structures specified with phase_list. The elastic constants are evaluated by finite displacement method, with a displacement of ±10-4. The parameter use_built_in can be set to True or False. When use_built_in = True, the Lava Wrapper generates all the deformed configurations and calculates the elastic constants, whereas when use_built_in = False, fix/deform is used in lammps to apply deformation, and a combination of IBRION, ISIF, NFREE tag (see default values in function definition) is specified in vasp allowing for calling the built-in subroutine to evaluate elastic constants. Besides elastic constants, other elastic properties such as bulk modulus, shear modulus and Poisson’s ratio are also calculated.

### Elastic properties phase_list = ["FCC", "HCP"] Lava_Calculator.get_elastic_properties(phase_list, mode=mode, use_built_in=False)

The output in Summary.dat is shown below:

FCC: C11 = 113.792 GPa C12 = 61.550 GPa C44 = 31.595 GPa Bulk modulus = 78.964 GPa Shear modulus = 26.121 GPa Poisson Ratio = 0.351 HCP: C11 = 116.117 GPa C12 = 55.404 GPa C13 = 45.209 GPa C33 = 69.833 GPa C44 = 11.185 GPa Bulk modulus = 75.641 GPa Shear modulus = 30.357 GPa Poisson Ratio = 0.323

  - Radial distribution function (RDF): Lammps only

def get_liquid_rdf(T_dict, lat_rep=[40,40,40], orz=[0,0,1], mode=None):

The following snippet in Lava_Wrapper.py calculates the RDF specified with T_dict, which is a dictionary with the keys corresponding to a phase and values corresponding to a list of temperature for that phase. This calculation is performed in Lammps mode only.

### Radial distribution function for liquid at different T (Lammps only) T_dict = {"FCC": range(300,1500,300)} Lava_Calculator.get_liquid_rdf(T_dict, mode=mode)

The output is written to Liquid_RDF_FCC_Lammps.dat. The corresponding plot Liquid _RDF_FCC_Lammps.jpeg is shown below in Fig. 3.

Fig. 3  RDF plot generated by Lava Wrapper.

  - Vacancy, interstitial formation energy

def get_vacancy_interstitial_energy(phase_list, NSW=100, IBRION=1, ISIF=2, lat_rep=None, orz=[0,0,1], lat_dim=[12,12,12], mode=None):

The following snippet in Lava_Wrapper.py calculates the vacancy and interstitial formation energy of given list of crystal structures specified with phase_list at 0 K. A lat_rep can be specified to replicate the unit cell, or lat_dim can be specified such that the unit cell is replicated to lat_dim.

### Vacancy/interstitial formation energy phase_list = ["FCC"] Lava_Calculator.get_vacancy_interstitial_energy(phase_list, lat_dim=[12,12,12], mode=mode)

The output in Summary.dat is shown below:

FCC: Vacancy formation energy: 0.681 eV Interstitial formation energy: 3.096 eV

  - Surface energy

def get_surface_energy(surface_dict, NSW=100, IBRION=1, ISIF=2, lat_rep=None, lat_dim=[3,3,20], vac=10, mode=None):

The following snippet in Lava_Wrapper.py calculates surface energy at 0 K specified with surface_list, which is a dictionary with the keys corresponding to a phase and values corresponding to a list of surfaces for that phase. Specifications of lat_rep and lat_dim is similar to that in section 3.6.

### Surface energy surface_dict = {"FCC": [(0,0,1), (1,1,0), (1,1,1)], "HCP": [(0,0,0,1), (1,1,-2,0), (1,0,-1,0)]} Lava_Calculator.get_surface_energy(surface_dict, mode=mode)

The output in Summary.dat is shown below:

(001) Surface energy: 943.580 mJ/m2 (110) Surface energy: 1006.016 mJ/m2 (111) Surface energy: 870.511 mJ/m2 (0001) Surface energy: 821.416 mJ/m2 (11-20) Surface energy: 982.039 mJ/m2 (10-10) Surface energy: 1142.438 mJ/m2

  - Thermal expansion: Lammps only

def get_thermal_expansion(T_dict, lat_rep=[10,10,10], orz=[0,0,1], mode=None):

The following snippet in Lava_Wrapper.py calculates temperature-dependent lattice constants. T_dict is a dictionary with the keys corresponding to a phase and values corresponding to a list of surfaces for that phase. This calculation is performed in Lammps mode only.

### Thermal expansion (Lammps only) T_dict = {"FCC": range(400,1000,200)} Lava_Calculator.get_thermal_expansion(T_dict, mode=mode)

The output is written to Thermal_expansion_FCC_Lammps.dat. The corresponding plot Thermal_expansion _FCC_Lammps.jpeg is shown below in Fig. 4.

Fig. 4  Thermal expansion plot generated by Lava Wrapper.

  - Bain path (1D and 2D)

def get_Bain_path(clat_start, clat_end, npoints, NSW=1, IBRION=1, ISIF=2, lat_rep=[1,1,1], orz=[0,0,1], mode=None): def get_Bain_path_2D(clat_start, clat_end, npoints_c, Vol_start, Vol_end, npoints_V, NSW=1, IBRION =1, ISIF=2, lat_rep=[1,1,1], orz=[0,0,1], mode=None):

The following snippet in Lava_Wrapper.py calculates volume-conserving 1-D Bain path and volume-non-conserving 2-D Bain path, where the energy of a FCC lattice is calculated for varying c/a ratio at constant volume (volume-conserving 1-D Bain path), or at varying c/a ratio and varying volume (volume-non-conserving 2-D Bain path).

### Volume-conserving Bain path npoints = 30 Lava_Calculator.get_Bain_path(0.5, 1.2, npoints, mode=mode) ### Volume-conserving Bain path (2D) npoints = 15 Lava_Calculator.get_Bain_path_2D(0.5, 1.2, npoints, 0.8, 1.2, npoints, mode=mode)

The output is written to Bain_Lammps.dat, and 2D_Bain_Lammps.dat.The corresponding plots: Bain_Lammps.jpeg and 2D_Bain_Lammps.jpeg are shown below in Fig. 5.

(a)  (b)

Fig. 5  (a) 1-D volume-conserving Bain path plot, (b) 2-D Bain path plot generated by Lava Wrapper.

  - Stacking fault energy (1D and 2D)

def get_stacking_fault(phase, sf_mesh, NSW=100, IBRION=1, ISIF=2, mode=None):

The following snippet in Lava_Wrapper.py calculates to get the energetics of the slip vs twin path for FCC phase. Currently, four Gamma surfaces system are supported:

FCC -> X: [11-2], Y: [1-10], Z:[111]

BCC -> X: [111], Y: [-110], Z: [11-2]

HCP -> X: [11-20], Y: [10-10], Z: [0001].

A5_Beta_Sn -> X: [001], Y: [010], Z: [100]

The corresponding slip systems are selected when phase is specified. Here, sf_mesh specifies the 2-D mesh used to sample the Gamma surface.

### Stacking fault energy: 2-D grid, very expensive for VASP phase = "FCC" sf_mesh = [18,18] Lava_Calculator.get_stacking_fault(phase, sf_mesh, mode=mode)

The output is written to 2D_Gamma_Lammps.dat, and 3D_Gamma_Lammps.dat.The corresponding plots: 2D_Gamma _Lammps.jpeg and 3D_Gamma_Lammps.jpeg are shown below in Fig. 6.

(a)   (b)

Fig. 6  (a) 2-D Gamma surface, (b) 3-D Gamma surface plot generated by Lava Wrapper.

Moreover, a generalized stacking fault energy curve (GSFE) can also be calculated.

def plot_GSFE_v2(blat,clat,phase,direction,input_X, input_Y, input_Z, output_file):

From the 2-D, 3-D gamma map, the GSFE along any direction within the surface can be obtained. This is available for all 4 gamma surfaces.

def get_general_stacking_fault(phase, sf_mesh, NSW=100, IBRION=1, ISIF=2, mode=None):

For FCC and BCC structures, a full slip path, as well as a twinning path achieved through successive slip of neighboring layers along [11-2] (FCC) and [111] (BCC) direction is calculated, with the following snippet in Lava_Wrapper.py. sf_mesh is the size of the mesh, wherein sf_mesh[0] needs to a multiplier of 3, and sf_mesh[1] needs to be 1. GSFE can be calculated for FCC or BCC phase.

### General stacking fault curve (full slip vs twin) phase = "FCC" sf_mesh = [24,1] Lava_Calculator.get_general_stacking_fault(phase, sf_mesh, mode=mode)

The output is written to GSFE_Lammps.dat, with the corresponding plot: GSFE_ Lammps.jpeg as shown below in Fig. 7.

Fig. 7  GSFE plot generated by Lava Wrapper.

  - Melting point: Lammps only

The following snippet in Lava_Wrapper.py determines the melting point for each phase in the phase_list corresponding to a phase with initial guess between [Tleft,Tright] and desired T_error. Specifications of lat_rep is similar to that in section 3.6.

def get_melting_point(phase_list, Tleft, Tright T_error, lat_rep=[10,10,50], orz=[0,0,1], mode=None):

The idea is to first construct a solid-liquid interface at the center of the simulation cell along the Z direction at the estimated melting point T_est=(T_left+T_right)/2, and equilibrate the system via an isenthalpic Nosé-Hoover style barostat (NPH). By default, LAVA creates this with a fully periodic simulation cell measuring 10x10x50 unit cells with the Z direction aligned along the 50 unit cell axis. On one side of the halfway mark, the cell is initially heated to the input temperature, T_est, and the other half is superheated to 1.2T_est+300 to form a liquid structure. Both domains then cooled to T_est. Under isenthalpic conditions, T_est>T_m means the solid is unstable and will melt, lowering the overall temperature as sensible heat becomes latent heat. Conversely, T_est<T_m will cause the liquid to solidify, raising the temperature as latent heat is released. In either scenario, the isenthalpic Nosé-Hoover style barostat brings the overall temperature closer to Tm. Therefore, the sign of the difference between the input temperature and output temperature will be positive below T_m and negative above T_m. LAVA leverages this by using the bisection method to converge to T_m. The simulation is terminated when the difference between the lower and upper bounds is less than T_error. The final melting point is given as T_m=T±T_error/2.

Fig. 7 Bisection algorithm to determine the melting temperature using LAMMPS for LAVA.

### Melting point using 2 phase method (Lammps only) ## get_melting_point(T, T_error): T is trial melting point, T_error is convergence criteria phase_list = ["FCC"] Lava_Calculator.get_melting_point(phase_list, 400,1000, 10, mode=mode)

Thus, if we start from a trial melting point between (T_left, T_right) of (400,1000) K, and set T_error to be 10 K, then we obtain the following output in Summary.dat. Five runs are required to achieve convergence to the melting point of 878.1 ± 10.0 K obtained by the Lava Wrapper. The bisection provides a consistent speed to convergence compared to other searching algorithm. The number of iterations can be predetermined based on the initial range and the convergence tolerance T_error.

Left T: 700.0 K, Right T: 1000.0 K, delta_T: 150.0 K Left T: 850.0 K, Right T: 1000.0 K, delta_T: 75.0 K Left T: 850.0 K, Right T: 925.0 K, delta_T: 37.5 K Left T: 850.0 K, Right T: 887.5 K, delta_T: 18.8 K Left T: 868.8 K, Right T: 887.5 K, delta_T: 9.4 K delta_T = 9.4 < 10.0 (T_error), convergence achieved. Melting point: 878.1 +- 10.0 K

The final melt in the last run is shown below in Fig. 8, where a well-defined solid-liquid interface at the center along Z direction can be seen. The dimension of the system, with a replication of 10 x 10 x 50, is ~ 4 x 4 x 20 nm, with 20k atoms.

Fig. 8  The final melt at 1039.2 K obtained with 2-phase method in Lava Wrapper. Atoms are colored with Common Neighbor Analysis (CNA).

  - Uniaxial deformation: Lammps only

def uniaxial_deform(phase_list, temp, deform_mode , rate, final_strain,lat_rep, orz=[0,0,1], mode=None):

The following snippet in Lava_Wrapper.py applies uniaxial deformation along desired directions (specified in the deform_mode) at a given temperature (temp), strain rate (rate), and final_strain. The values for the deform_mode are defined such as: 1x, 2y, 3z, 4yz, 5xz, 6 yz directions. The uniaxial deformation along desired direction is performed via the fix/deform command.

# Uniaxial deformation (Lammps_serial only) # Perform uniaxial deformation along desired direction with erate/trate via a fix/deform command # deform_mode indicates which deformation mode is performed # 1=x 2=y,3=z, 4=yz, 5=xz, 6=xy phase_list = ["A5_Beta_Sn","A5_Gamma_Sn"] temp = 300 lat_rep=[15,15,15] deform_mode = [1,2,3,4,5,6] rate = 0.01 final_strain = 0.25 Lava_Calculator.uniaxial_deform(phase_list,temp,deform_mode,rate, final_strain, lat_rep,mode=mode)

The output is written to Stress_strain_*deform_mode*_*phase*.txt.The corresponding plots: Stress_strain_1_A5_Beta_Sn.jpeg are shown below in Fig. 9.

Fig. 9  Stress-strain curve beta phase (x) final melt at 300 K obtained with 2-phase method in Lava Wrapper. Atoms are colored with Common Neighbor Analysis (CNA).

Appendix: Lava_Generator.py as a stand-alone configuration generator

Lava_Generator.py itself can be used as a stand-alone python code to generate configurations, and thus can be useful outside of Lava Wrapper as well.

The following call generates FCC lattice with a lattice constant of 4.05, and c/a ratio of 1.00, and Z orientation of [111], replicated by 2 x 2 x 2 times along X, Y, Z direction:

python Generator.py latt FCC 4.05 1.00 orz 1 1 1 rep 2 2 2

Adding a “vac 10” to the above call will generate a (111) surface with a 10 Å vacuum (half on each side):

python Generator.py latt FCC 4.05 1.00 orz 1 1 1 rep 2 2 2 vac 10

The following call adds 1 vacancy to the input data file Temp.data:

python Generator.py Temp.data vacancy 1

Substitute vacancy with interstitial will add 1 interstitial atom to the input data file Temp.data:

python Generator.py Temp.data interstitial 1

The following call generates a 24x24 stacking fault mesh structures, based on a lattice constant of 4.05.

python Generator.py Stacking_Fault 4.05 24 24

Reference

1.	Mishin, Y., et al., Interatomic potentials for monoatomic metals from experimental data and ab initio calculations. Physical Review B, 1999. 59(5): p. 3393-3407.